# Cross-Compiler Build Procedure

Using the LLVM Framework  (Clang + LLD + LLDB + compiler-rt)

Targets: ARM Cortex-A (AArch32/AArch64)  |  Cortex-M (Thumb-2 bare-metal)  |  uG24 (custom 8-bit)

| Document Version | 2.1 |
| --- | --- |
| LLVM Version | 17.x Stable (llvmorg-17.0.6) |
| Host Platform | x86_64 Linux (Ubuntu 22.04 LTS recommended) |
| Target Triples | armv7-linux-gnueabihf ¦ aarch64-linux-gnu ¦ arm-none-eabi ¦ ug24-unknown-none-eabi |
| Scope | Full toolchain: clang, clang++, lld, lldb, compiler-rt, plus the uG24 back end, runtime and simulator |
| Supersedes | v2.0 — see the change list in section 12.1 |

NOTE

Every command in this document was executed on the reference machine and its output captured. Expected output is shown after each step in a shaded block. Revision 2.1 re-ran all of them against the merged repository.

## 1.  Overview

This procedure produces five compilers. Stage 1 is a native x86 clang used only as a build tool; the four that follow are the deliverables.

| Compiler | Triple | Use | Runs on |
| --- | --- | --- | --- |
| stage1 | x86_64-linux-gnu | Builds the other four | host |
| toolchain-armhf | armv7-linux-gnueabihf | 32-bit ARM Linux userspace | qemu-arm |
| toolchain-arm64 | aarch64-linux-gnu | 64-bit ARM Linux userspace | qemu-aarch64 |
| toolchain-none-eabi | arm-none-eabi | Cortex-M bare metal | qemu-system-arm |
| build-ug24 | ug24-unknown-none-eabi | Custom 8-bit processor | ug24sim |

### 1.1  What to carry to the new machine

| Item | Contains | Size |
| --- | --- | --- |
| Git repository llvm-project-ug24 | LLVM monorepo including the uG24 back end (31 files), the lld back end, the clang target, ABI and driver, the ELF machine and relocation definitions, and — since the workspace was merged in — the uG24 runtime, simulator, tests and documentation. | ~2 GB |
| Nothing else — | Revision 2.0 carried a separate ug24-support-files.tar.gz. There is no bundle any more: the runtime, simulator, tests, helper scripts, the CMake toolchain file and the documentation all live in the repository, so a clone is the whole delivery and the compiler can never drift from the runtime it needs. | — |

### 1.2  Final directory layout

LAYOUT

$HOME/llvm-arm-cross/

├── llvm-project/           # the clone -- everything uG24 is inside it

│   ├── llvm/lib/Target/UG24/     # the back end

│   ├── clang/lib/CodeGen/Targets/UG24.cpp   # the C ABI

│   ├── ug24-runtime/  ug24-sim/  ug24-tests/  docs/

│   ├── toolchain-ug24.cmake       ug24-setup.sh

│   └── build-ug24/                # section 8

│

├── sysroot-none-eabi/      # section 4.4

├── arm-tests/              # ARM helper scripts

│

├── stage1/                 # section 5

├── toolchain-armhf/        # section 6.1

├── toolchain-arm64/        # section 6.2

└── toolchain-none-eabi/    # section 6.3

## 2.  Host System Prerequisites

### 2.1  Hardware and OS requirements

| Item | Minimum | Recommended |
| --- | --- | --- |
| Host OS | Ubuntu 22.04 / Debian 12 | Ubuntu 22.04 LTS |
| Architecture | x86_64 (amd64) | x86_64 (amd64) |
| RAM | 16 GB | 32 GB |
| Free disk | 80 GB | 120 GB |
| CPU cores | 4 | 8 or more |
| Internet | Required (clone ~2 GB) | Required |

### 2.2  Install build dependencies

SHELL

sudo apt update && sudo apt install -y \

build-essential git cmake ninja-build python3 python3-pip \

pkg-config libssl-dev zlib1g-dev libedit-dev libxml2-dev \

libffi-dev swig doxygen \

gcc-arm-linux-gnueabihf g++-arm-linux-gnueabihf \

gcc-aarch64-linux-gnu   g++-aarch64-linux-gnu \

gcc-arm-none-eabi       binutils-arm-none-eabi \

libc6-dev-armhf-cross   linux-libc-dev-armhf-cross \

qemu-user-static        qemu-system-arm  xxd

NOTE

The gcc-* cross packages are not used to compile your code. They supply the sysroots — target headers and libc — and the libgcc and crtbegin objects that clang links against for hosted targets.

swig and libedit-dev are required by LLDB. qemu-system-arm is needed to execute bare-metal images.

### 2.3  Verify tool versions

SHELL

cmake --version    # must be >= 3.20

ninja --version    # must be >= 1.10

python3 --version  # must be >= 3.6

EXPECTED OUTPUT

cmake version 3.28.3

1.11.1

Python 3.12.3

If CMake is too old: pip3 install --upgrade cmake

### 2.4  Environment variables

Add these to ~/.bashrc so they survive new shells.

~/.BASHRC

export LLVM_WORK=$HOME/llvm-arm-cross

export SYSROOT_ARMHF=/usr/arm-linux-gnueabihf

export SYSROOT_ARM64=/usr/aarch64-linux-gnu

export SYSROOT_NONE=$LLVM_WORK/sysroot-none-eabi

export PATH=$LLVM_WORK/stage1/bin:$PATH

SHELL

source ~/.bashrc && echo "$LLVM_WORK | $SYSROOT_ARMHF"

EXPECTED OUTPUT

/home/<user>/llvm-arm-cross | /usr/arm-linux-gnueabihf

CAUTION

AN UNSET VARIABLE FAILS SILENTLY. --sysroot=$SYSROOT_ARMHF with the variable unset expands to --sysroot=, which clang accepts and ignores. The result is a confusing 'stdio.h not found' much later. Always confirm the echo above before building.

## 3.  Obtain the Source Code

### 3.1  Clone the repository

This is not upstream LLVM. It is the monorepo with the uG24 back end already merged in, and since revision 2.1 the uG24 runtime, simulator and tests as well, so one clone gives all five compilers and everything needed to run uG24 code.

SHELL

mkdir -p $LLVM_WORK && cd $LLVM_WORK

git clone --depth 1 \

git@github.com:Jayanthkumar1233/llvm-project-ug24.git llvm-project

cd llvm-project && git log --oneline -1

EXPECTED OUTPUT

b06ba2a38 Re-check the specification queries against the toolchain as built

### 3.2  Confirm the uG24 workspace arrived

SHELL

cd $LLVM_WORK/llvm-project

ls -d ug24-runtime ug24-sim ug24-tests docs ug24-setup.sh

EXPECTED OUTPUT

docs  ug24-runtime  ug24-setup.sh  ug24-sim  ug24-tests

### 3.3  Confirm the uG24 target arrived

SHELL

ls llvm-project/llvm/lib/Target/UG24/ | wc -l

ls llvm-project/lld/ELF/Arch/UG24.cpp

grep -c EM_UG24 llvm-project/llvm/include/llvm/BinaryFormat/ELF.h

EXPECTED OUTPUT

31

llvm-project/lld/ELF/Arch/UG24.cpp

1

## 4.  Prepare the Sysroots

A sysroot is a directory tree holding the target's headers and libraries. Clang uses --sysroot to find them without touching host paths.

### 4.1  Sysroot options

| Method | Best for | How to obtain |
| --- | --- | --- |
| Option A — installed GCC sysroot | Quick start; already on the host | /usr/arm-linux-gnueabihf |
| Option B — debootstrap | A clean minimal Debian rootfs | sudo debootstrap --arch armhf ... |
| Option C — newlib copy | Bare metal, no OS | cp /usr/lib/arm-none-eabi/... |

### 4.2  Option A — use the installed GCC sysroot  (recommended)

SHELL

# 32-bit ARM hard-float

export SYSROOT_ARMHF=/usr/arm-linux-gnueabihf

# 64-bit ARM

export SYSROOT_ARM64=/usr/aarch64-linux-gnu

# Verify headers and libraries are present

ls $SYSROOT_ARMHF/include/    # Expect: stdio.h, stdlib.h, ...

ls $SYSROOT_ARMHF/lib/        # Expect: libc.so, libm.so, ...

EXPECTED OUTPUT

aio.h  aliases.h  alloca.h  a.out.h  argp.h  argz.h  ...

libc.a  libc.so  libc.so.6  libm.a  libm.so  ...

### 4.3  Option B — build a clean sysroot with debootstrap

SHELL

export SYSROOT_ARMHF=$LLVM_WORK/sysroot-armhf

mkdir -p $SYSROOT_ARMHF

sudo debootstrap --arch=armhf \

--include=libstdc++-12-dev,libc6-dev,libgcc-12-dev \

bullseye $SYSROOT_ARMHF http://deb.debian.org/debian

# Fix absolute symlinks, which break when read from the host

cd $SYSROOT_ARMHF

find . -type l | while read link; do

target=$(readlink "$link")

[ "${target#/}" != "$target" ] && ln -sf "${SYSROOT_ARMHF}${target}" "$link"

done

# MUST verify — a partial debootstrap looks like success

ls $SYSROOT_ARMHF/usr/include/stdio.h

find $SYSROOT_ARMHF -name 'libc.so*' | head -2

CAUTION

OPTION B FAILED ON THE PREVIOUS BUILD — VERIFY IT. The debootstrap died after downloading the package index, leaving a directory with five files and no libc. debootstrap exits without an obvious error, and the failure only surfaces later as unresolvable link errors.

If the two checks above do not both succeed, delete the directory and use Option A.

### 4.4  Option C — bare-metal sysroot (arm-none-eabi)

For Cortex-M there is no OS; the sysroot is newlib headers and archives.

SHELL

export SYSROOT_NONE=$LLVM_WORK/sysroot-none-eabi

mkdir -p $SYSROOT_NONE/include $SYSROOT_NONE/lib

cp -r /usr/lib/arm-none-eabi/include/* $SYSROOT_NONE/include/

cp    /usr/lib/arm-none-eabi/lib/*.a    $SYSROOT_NONE/lib/

ls $SYSROOT_NONE/include/ | head -5

ls $SYSROOT_NONE/lib/*.a | head -4 | xargs -n1 basename

EXPECTED OUTPUT

alloca.h  _ansi.h  argz.h  ar.h  assert.h

libc.a

libc_nano.a

libg.a

libgloss-linux.a

## 5.  Stage 1 — Build a Native Host Clang/LLVM

Stage 1 is an x86_64 clang with the ARM and AArch64 back ends enabled. It is used to build every later stage: clang building clang produces a better result than the system GCC, and guarantees all stages use one front end.

Estimated time: 40–90 minutes on 8 cores.

### STEP 5.1  Create the Stage 1 build directory

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-stage1 && cd build-stage1

### STEP 5.2  Configure Stage 1 with CMake

SHELL

cmake -G Ninja ../llvm \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/stage1 \

-DLLVM_TARGETS_TO_BUILD="X86;ARM;AArch64" \

-DLLVM_ENABLE_PROJECTS="clang;lld" \

-DLLVM_ENABLE_RUNTIMES="" \

-DLLVM_BUILD_TOOLS=ON \

-DLLVM_INCLUDE_TESTS=OFF \

-DLLVM_INCLUDE_EXAMPLES=OFF \

-DCLANG_DEFAULT_LINKER=lld \

-DLLVM_PARALLEL_LINK_JOBS=1

| CMake flag | Purpose |
| --- | --- |
| LLVM_TARGETS_TO_BUILD="X86;ARM;AArch64" | Host (X86) plus both ARM back ends |
| LLVM_ENABLE_PROJECTS="clang;lld" | Clang front end and LLD linker. LLDB is added in Stage 2 only |
| LLVM_ENABLE_RUNTIMES="" | No compiler-rt or libcxx here — Stage 1 is a build tool, not a shipped toolchain |
| LLVM_BUILD_TOOLS=ON | Builds llvm-config, llvm-ar, llvm-nm and the rest, which later stages need |
| CMAKE_BUILD_TYPE=Release | Optimised build. Never use Debug: 3x larger, much slower |
| LLVM_PARALLEL_LINK_JOBS=1 | Each link job needs several GB. Parallel links get OOM-killed on 16 GB |
| CMAKE_INSTALL_PREFIX=.../stage1 | Separate install dir keeps Stage 1 isolated from Stage 2 |

### STEP 5.3  Build and install Stage 1

SHELL

ninja -j$(nproc) install

# Verify installation

$LLVM_WORK/stage1/bin/clang --version

$LLVM_WORK/stage1/bin/llvm-config --targets-built

EXPECTED OUTPUT

clang version 17.0.6 (https://github.com/llvm/llvm-project.git 6009708b...)

Target: x86_64-unknown-linux-gnu

Thread model: posix

X86 ARM AArch64

SHELL

# Stage 1 must be on PATH for every step that follows

export PATH=$LLVM_WORK/stage1/bin:$PATH

## 6.  Stage 2 — Build the ARM Cross-Compilers

Each target gets its own independent build directory. All three use the Stage 1 clang as the compiler.

### 6.1  Target A — armv7-linux-gnueabihf  (32-bit ARM Linux)

Estimated time: 60–120 minutes on 8 cores.

### STEP 6.1.1  Configure

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-armhf && cd build-armhf

cmake -G Ninja ../llvm \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/toolchain-armhf \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_CXX_COMPILER=$LLVM_WORK/stage1/bin/clang++ \

-DLLVM_TARGETS_TO_BUILD="ARM" \

-DLLVM_ENABLE_PROJECTS="clang;lld;lldb" \

-DLLVM_ENABLE_RUNTIMES="" \

-DLLVM_DEFAULT_TARGET_TRIPLE="armv7-linux-gnueabihf" \

-DLLVM_HOST_TRIPLE="x86_64-linux-gnu" \

-DCLANG_DEFAULT_LINKER=lld \

-DDEFAULT_SYSROOT=$SYSROOT_ARMHF \

-DLLVM_PARALLEL_LINK_JOBS=1 \

-DLLVM_INCLUDE_TESTS=OFF \

-DLLVM_INCLUDE_EXAMPLES=OFF \

-DCMAKE_BUILD_WITH_INSTALL_RPATH=ON

NOTE

CMAKE_BUILD_WITH_INSTALL_RPATH=ON is required whenever lldb is in LLVM_ENABLE_PROJECTS. LLDB links against shared LLVM libraries; without this flag the installed lldb keeps an RPATH pointing into the build directory and stops working once that directory is deleted.

### STEP 6.1.2  Build and install

SHELL

ninja -j$(nproc) install

### STEP 6.1.3  Verify the toolchain itself

Confirm the compiler before compiling anything with it.

SHELL

$LLVM_WORK/toolchain-armhf/bin/clang \

--target=armv7-linux-gnueabihf \

--sysroot=$SYSROOT_ARMHF \

--print-target-triple

$LLVM_WORK/toolchain-armhf/bin/llvm-config --targets-built

$LLVM_WORK/toolchain-armhf/bin/lldb --version | head -1

EXPECTED OUTPUT

armv7-unknown-linux-gnueabihf

ARM

lldb version 17.0.6 (https://github.com/llvm/llvm-project.git revision 6009708b...)

NOTE

THE TRIPLE GAINS 'unknown'. Clang normalises armv7-linux-gnueabihf into the four-field form armv7-unknown-linux-gnueabihf. That is correct — the vendor field is simply made explicit. Rev 1.1 of this document expected the three-field form.

### 6.2  Target B — aarch64-linux-gnu  (64-bit ARM Linux)

Estimated time: 60–120 minutes on 8 cores.

### STEP 6.2.1  Configure, build and install

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-arm64 && cd build-arm64

cmake -G Ninja ../llvm \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/toolchain-arm64 \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_CXX_COMPILER=$LLVM_WORK/stage1/bin/clang++ \

-DLLVM_TARGETS_TO_BUILD="AArch64" \

-DLLVM_ENABLE_PROJECTS="clang;lld;lldb" \

-DLLVM_ENABLE_RUNTIMES="" \

-DLLVM_DEFAULT_TARGET_TRIPLE="aarch64-linux-gnu" \

-DLLVM_HOST_TRIPLE="x86_64-linux-gnu" \

-DCLANG_DEFAULT_LINKER=lld \

-DDEFAULT_SYSROOT=$SYSROOT_ARM64 \

-DLLVM_PARALLEL_LINK_JOBS=1 \

-DLLVM_INCLUDE_TESTS=OFF \

-DLLVM_INCLUDE_EXAMPLES=OFF \

-DCMAKE_BUILD_WITH_INSTALL_RPATH=ON

ninja -j$(nproc) install

### STEP 6.2.2  Verify

SHELL

$LLVM_WORK/toolchain-arm64/bin/clang \

--target=aarch64-linux-gnu \

--sysroot=$SYSROOT_ARM64 \

--print-target-triple

$LLVM_WORK/toolchain-arm64/bin/llvm-config --targets-built

EXPECTED OUTPUT

aarch64-unknown-linux-gnu

AArch64

### 6.3  Target C — arm-none-eabi  (Cortex-M Bare-Metal)

Bare-metal targets require a different configuration: no OS, no dynamic linking, and compiler-rt builtins instead of a full runtime library.

CAUTION

COMPILER-RT MUST NOT BE INCLUDED IN LLVM_ENABLE_RUNTIMES HERE. If it is, the runtimes sub-build auto-selects -mfloat-abi=soft, which disables the VFP registers. The VFP assembly builtins (fixunssfsivfp.S and friends) then fail with 'instruction requires: VFP2'.

The COMPILER_RT_* flags below are CMake cache variables. They are harmless here — nothing reads them while compiler-rt is not being configured — and are kept so the same cache serves section 7.2, where the builtins are actually built with the correct -mfpu and -mfloat-abi flags.

### STEP 6.3.1  Configure, build and install

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-none-eabi && cd build-none-eabi

cmake -G Ninja ../llvm \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/toolchain-none-eabi \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_CXX_COMPILER=$LLVM_WORK/stage1/bin/clang++ \

-DLLVM_TARGETS_TO_BUILD="ARM" \

-DLLVM_ENABLE_PROJECTS="clang;lld" \

-DLLVM_ENABLE_RUNTIMES="" \

-DLLVM_DEFAULT_TARGET_TRIPLE="arm-none-eabi" \

-DLLVM_HOST_TRIPLE="x86_64-linux-gnu" \

-DCLANG_DEFAULT_LINKER=lld \

-DCOMPILER_RT_BAREMETAL_BUILD=ON \

-DCOMPILER_RT_BUILD_BUILTINS=ON \

-DCOMPILER_RT_BUILD_SANITIZERS=OFF \

-DCOMPILER_RT_BUILD_XRAY=OFF \

-DCOMPILER_RT_BUILD_LIBFUZZER=OFF \

-DCOMPILER_RT_BUILD_PROFILE=OFF \

-DLLVM_PARALLEL_LINK_JOBS=1 \

-DLLVM_INCLUDE_TESTS=OFF

ninja -j$(nproc) install

LLDB is deliberately absent here: there is no OS to attach to, and remote debugging a Cortex-M part goes through a JTAG probe rather than LLDB's process plugins.

### STEP 6.3.2  Verify

SHELL

$LLVM_WORK/toolchain-none-eabi/bin/clang \

--target=arm-none-eabi --print-target-triple

$LLVM_WORK/toolchain-none-eabi/bin/llvm-config --targets-built

EXPECTED OUTPUT

arm-none-unknown-eabi

ARM

## 7.  Build compiler-rt Builtins

compiler-rt supplies the low-level routines clang emits calls to — 64-bit division, soft-float emulation, stack protection — replacing libgcc. It must be cross-compiled once per target architecture.

### 7.1  compiler-rt for armv7-linux-gnueabihf

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-rt-armhf && cd build-rt-armhf

# Pick up whichever cross-GCC version is installed

export GCC_ARM_VER=$(ls /usr/lib/gcc-cross/arm-linux-gnueabihf/ | tail -1)

echo "using cross GCC $GCC_ARM_VER"

cmake -G Ninja ../compiler-rt \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/toolchain-armhf \

-DLLVM_CMAKE_DIR=$LLVM_WORK/stage1/lib/cmake/llvm \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_CXX_COMPILER=$LLVM_WORK/stage1/bin/clang++ \

-DCMAKE_ASM_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_C_COMPILER_TARGET="armv7-linux-gnueabihf" \

-DCMAKE_CXX_COMPILER_TARGET="armv7-linux-gnueabihf" \

-DCMAKE_ASM_COMPILER_TARGET="armv7-linux-gnueabihf" \

-DCMAKE_C_COMPILER_EXTERNAL_TOOLCHAIN=/usr \

-DCMAKE_CXX_COMPILER_EXTERNAL_TOOLCHAIN=/usr \

-DCMAKE_SYSROOT=/ \

-DCMAKE_C_FLAGS="--gcc-toolchain=/usr -march=armv7-a -mfloat-abi=hard" \

-DCMAKE_CXX_FLAGS="--gcc-toolchain=/usr -march=armv7-a -mfloat-abi=hard" \

-DCMAKE_ASM_FLAGS="--gcc-toolchain=/usr -march=armv7-a -mfloat-abi=hard" \

-DCOMPILER_RT_DEFAULT_TARGET_TRIPLE="armv7-linux-gnueabihf" \

-DCOMPILER_RT_BUILD_BUILTINS=ON \

-DCOMPILER_RT_BUILD_SANITIZERS=ON \

-DCOMPILER_RT_BUILD_XRAY=OFF

ninja -j$(nproc) install

ls $LLVM_WORK/toolchain-armhf/lib/linux/ | grep builtins

EXPECTED OUTPUT

using cross GCC 13

...

libclang_rt.builtins-armhf.a

GCC_ARM_VER records the installed cross-GCC version so the recipe does not hard-code 13. An empty value means the gcc-arm-linux-gnueabihf package is missing.

### 7.2  compiler-rt builtins for arm-none-eabi  (Cortex-M4) — OPTIONAL

NOTE

SET THIS SECTION ASIDE ON A FIRST BUILD. It is not required to have a working bare-metal toolchain. Section 10.2 compiles with clang and links with arm-none-eabi-gcc, which brings its own libgcc — that path works whether or not 7.2 has been done.

Do 7.2 only when you want to link bare-metal images with ld.lld and no GNU tools at all, which section 10.2.1 demonstrates.

CAUTION

THIS SECTION IS CORRECTED — REV 1.1 DID NOT WORK. The v1.1 recipe pointed CMake at ../compiler-rt without forcing the target, so COMPILER_RT_DEFAULT_TARGET_TRIPLE stayed at x86_64-unknown-linux-gnu. The build silently produced armhf libraries in build-rt-m4 and nothing bare-metal was ever installed. Linking then failed with 'undefined symbol: __aeabi_ldivmod'.

Two changes fix it: point CMake at compiler-rt/lib/builtins rather than all of compiler-rt, and set COMPILER_RT_DEFAULT_TARGET_ONLY=ON so the triple is taken from CMAKE_C_COMPILER_TARGET. Do NOT also set COMPILER_RT_DEFAULT_TARGET_TRIPLE — the two conflict and configuration fails.

SHELL

cd $LLVM_WORK/llvm-project

mkdir -p build-rt-m4 && cd build-rt-m4

cmake -G Ninja ../compiler-rt/lib/builtins \

-DCMAKE_BUILD_TYPE=Release \

-DCMAKE_INSTALL_PREFIX=$LLVM_WORK/toolchain-none-eabi/lib/clang/17 \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_ASM_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_C_COMPILER_TARGET=armv7em-none-eabi \

-DCMAKE_ASM_COMPILER_TARGET=armv7em-none-eabi \

-DCMAKE_TRY_COMPILE_TARGET_TYPE=STATIC_LIBRARY \

-DCMAKE_C_FLAGS="-mcpu=cortex-m4 -mthumb -mfpu=fpv4-sp-d16 -mfloat-abi=hard" \

-DCMAKE_ASM_FLAGS="-mcpu=cortex-m4 -mthumb -mfpu=fpv4-sp-d16 -mfloat-abi=hard" \

-DCOMPILER_RT_DEFAULT_TARGET_ONLY=ON \

-DCOMPILER_RT_BAREMETAL_BUILD=ON \

-DCOMPILER_RT_OS_DIR=baremetal \

-DCOMPILER_RT_BUILD_BUILTINS=ON \

-DCOMPILER_RT_BUILD_SANITIZERS=OFF \

-DCOMPILER_RT_BUILD_XRAY=OFF \

-DCOMPILER_RT_BUILD_LIBFUZZER=OFF \

-DCOMPILER_RT_BUILD_PROFILE=OFF \

-DCOMPILER_RT_BUILD_MEMPROF=OFF \

-DCOMPILER_RT_BUILD_ORC=OFF \

-DLLVM_CONFIG_PATH=$LLVM_WORK/stage1/bin/llvm-config

ninja -j$(nproc) install

EXPECTED OUTPUT

[186/186] Linking C static library lib/baremetal/libclang_rt.builtins-armv7em.a

### STEP 7.2.1  Verify the builtins are found by the driver

SHELL

$LLVM_WORK/toolchain-none-eabi/bin/clang --target=armv7em-none-eabi \

-rtlib=compiler-rt -print-libgcc-file-name

EXPECTED OUTPUT

.../toolchain-none-eabi/lib/clang/17/lib/baremetal/libclang_rt.builtins-armv7em.a

The printed path must be a file that exists. If it prints a path under lib/linux/ instead, COMPILER_RT_OS_DIR was not applied.

## 8.  Build the uG24 Cross-Compiler

uG24 is a custom 8-bit processor, not an upstream LLVM back end. The repository already contains the code generator, assembler, disassembler, ELF writer, lld back end, clang target and clang driver.

Estimated time: 60–120 minutes on 8 cores.

### STEP 8.1  Configure and build

SHELL

cd $LLVM_WORK/llvm-project

cmake -G Ninja -S llvm -B $LLVM_WORK/build-ug24 \

-DCMAKE_BUILD_TYPE=Release \

-DLLVM_TARGETS_TO_BUILD="UG24;AArch64;ARM" \

-DLLVM_ENABLE_PROJECTS="clang;lld" \

-DLLVM_ENABLE_RUNTIMES="" \

-DCMAKE_C_COMPILER=$LLVM_WORK/stage1/bin/clang \

-DCMAKE_CXX_COMPILER=$LLVM_WORK/stage1/bin/clang++ \

-DLLVM_PARALLEL_LINK_JOBS=1

cd $LLVM_WORK

ninja -C build-ug24 -j$(nproc) \

clang lld llc llvm-mc llvm-objdump llvm-readobj llvm-nm llvm-ar llvm-size llvm-dwarfdump FileCheck count not

NOTE

DO NOT ASK NINJA FOR llvm-lit. It is not a build target in any LLVM 17 tree: llvm/utils/llvm-lit/CMakeLists.txt only configure_file()s the script. FileCheck, count and not are real ninja targets and are what llvm-lit needs to run.

llvm-size and llvm-dwarfdump are new in revision 2.1. The first reports code size; the second reads uG24 objects now that the relocation resolver knows the target, where before it warned once per relocation.

ARM and AArch64 are included so this one build can also run the upstream regression suites, confirming that the shared-file changes (ELF.h, Triple.cpp, lld) broke nothing. This build is used in place, not installed.

### STEP 8.2  Verify the target is registered

SHELL

$LLVM_WORK/build-ug24/bin/clang --print-targets | grep ug24

EXPECTED OUTPUT

ug24       - uG24 8-bit Microprocessor

### STEP 8.3  Build the runtime, simulator and tests

The uG24 compiler on its own cannot link a program: it needs a startup file, a support library, a linker script and headers, none of which exist until they are built WITH that compiler. ug24-setup.sh at the top of the repository does that bootstrapping. It is no longer reproduced in full here — it ships with the clone. Its head, which is the part worth knowing, is how it finds the build tree:

UG24-SETUP.SH

ROOT=$(cd "$(dirname "$0")" && pwd)

BUILD=${1:-build-ug24}

if [ -n "$UG24_BUILD" ]; then

BUILD_DIR=$UG24_BUILD

elif [ -x "$ROOT/$BUILD/bin/clang" ]; then

BUILD_DIR="$ROOT/$BUILD"

elif [ -x "$ROOT/../$BUILD/bin/clang" ]; then

BUILD_DIR=$(cd "$ROOT/../$BUILD" && pwd)

else

echo "error: no uG24 clang in $ROOT/$BUILD/bin" >&2

exit 1

fi

It performs three steps in order:

1.  Runtime. Calls ug24-runtime/build-runtime.sh, which assembles crt0.s and ug24_setjmp.s with llvm-mc, compiles ug24_builtins.c, ug24_int64.c, ug24_float.c, ug24_io.c, ug24_stdio.c and ug24_stdlib.c with the new clang, archives them into libug24.a, and copies those with ug24.ld and the headers into build-ug24/lib/ug24/ — the directory the clang driver searches.

2.  Simulator. Compiles ug24-sim/ug24sim.c with the HOST compiler, since it runs on x86.

3.  Tests. Runs the 40-case end-to-end suite: compile, link, execute, compare against expected values. The fifteen-program acceptance suite is run separately — see section 10.4.

SHELL

cd $LLVM_WORK/llvm-project

./ug24-setup.sh

EXPECTED OUTPUT

==> building the uG24 runtime (crt0, libug24, linker script, headers)

uG24 runtime installed in /home/<user>/llvm-arm-cross/build-ug24/lib/ug24

==> building the simulator

/home/<user>/llvm-arm-cross/llvm-project/ug24-sim/ug24sim

==> running the end-to-end test suite

halted after 4084 instructions

pc=0068 sp=fefe ra=0066 psw=0090

...

cases run: 40, failures: 0

PASS

uG24 toolchain ready.

### 8.4  What build-runtime.sh installs

| File | Built from | Purpose |
| --- | --- | --- |
| lib/ug24/crt0.o | crt0.s | Vector table, sets SP, clears PSW.DP, copies .data, zeroes .bss, calls main |
| lib/ug24/libug24.a | ug24_builtins.c ug24_int64.c ug24_float.c ug24_io.c ug24_stdio.c ug24_stdlib.c ug24_setjmp.s | 8- to 64-bit shifts, multiply, divide; single-precision soft float; mem*/str*; the printf family; malloc and friends; setjmp/longjmp |
| lib/ug24/ug24.ld | ug24.ld | Memory layout, the vector table, the peripheral window, the heap/stack split |
| lib/ug24/include/ug24.h | include/ug24.h stdio.h stdlib.h string.h setjmp.h | Peripheral and interrupt-controller registers, print helpers, and the four standard headers the runtime implements |

NOTE

WHY THE RUNTIME IS BUILT WITH -ffreestanding -fno-builtin. Without those flags the optimiser recognises the shift and memcpy loops inside the helpers and rewrites them as calls to the very functions being defined — producing infinite recursion at run time. build-runtime.sh passes both.

## 9.  Wrapper Scripts and CMake Toolchain Files

Wrappers encode the target triple, sysroot and GCC installation so the toolchain behaves as a drop-in replacement for the GNU ARM cross-compiler.

### 9.1  Shell wrapper scripts (armv7-linux-gnueabihf)

CAUTION

THE HEREDOC DELIMITER MUST BE QUOTED. Use << 'WRAPPER' with quotes. Without them the shell expands $(dirname "$0") and "$@" WHILE WRITING THE FILE, producing a wrapper that execs /clang and drops all arguments. That is exactly what happened in the previous build.

SHELL

TBIN=$LLVM_WORK/toolchain-armhf/bin

# C compiler wrapper

cat > $TBIN/arm-linux-gnueabihf-clang << 'WRAPPER'

#!/bin/sh

exec "$(dirname "$0")/clang" \

--target=armv7-linux-gnueabihf \

--sysroot=/usr/arm-linux-gnueabihf \

--gcc-toolchain=/usr \

-fuse-ld=lld "$@"

WRAPPER

chmod +x $TBIN/arm-linux-gnueabihf-clang

# C++ compiler wrapper

cat > $TBIN/arm-linux-gnueabihf-clang++ << 'WRAPPER'

#!/bin/sh

exec "$(dirname "$0")/clang++" \

--target=armv7-linux-gnueabihf \

--sysroot=/usr/arm-linux-gnueabihf \

--gcc-toolchain=/usr \

-fuse-ld=lld "$@"

WRAPPER

chmod +x $TBIN/arm-linux-gnueabihf-clang++

Repeat for arm64, substituting toolchain-arm64, aarch64-linux-gnu and /usr/aarch64-linux-gnu.

### STEP 9.1.1  Verify the wrapper

SHELL

$LLVM_WORK/toolchain-armhf/bin/arm-linux-gnueabihf-clang \

-static -o /tmp/w.elf /tmp/hello.c

qemu-arm /tmp/w.elf

EXPECTED OUTPUT

Hello from ARM!

### 9.2  CMake toolchain file (ARM)

TOOLCHAIN-ARMHF.CMAKE

set(CMAKE_SYSTEM_NAME      Linux)

set(CMAKE_SYSTEM_PROCESSOR arm)

set(TRIPLE  armv7-linux-gnueabihf)

set(LLVMBIN $ENV{LLVM_WORK}/toolchain-armhf/bin)

set(CMAKE_C_COMPILER   ${LLVMBIN}/clang)

set(CMAKE_CXX_COMPILER ${LLVMBIN}/clang++)

set(CMAKE_LINKER       ${LLVMBIN}/ld.lld)

set(CMAKE_AR           ${LLVMBIN}/llvm-ar)

set(CMAKE_RANLIB       ${LLVMBIN}/llvm-ranlib)

set(CMAKE_STRIP        ${LLVMBIN}/llvm-strip)

set(CMAKE_C_COMPILER_TARGET   ${TRIPLE})

set(CMAKE_CXX_COMPILER_TARGET ${TRIPLE})

set(CMAKE_SYSROOT             /usr/arm-linux-gnueabihf)

set(CMAKE_C_FLAGS_INIT   "--gcc-toolchain=/usr")

set(CMAKE_CXX_FLAGS_INIT "--gcc-toolchain=/usr")

set(CMAKE_FIND_ROOT_PATH_MODE_PROGRAM NEVER)

set(CMAKE_FIND_ROOT_PATH_MODE_LIBRARY ONLY)

set(CMAKE_FIND_ROOT_PATH_MODE_INCLUDE ONLY)

### 9.3  CMake toolchain file (uG24)

toolchain-ug24.cmake sits at the top of the repository and needs no editing. Point it at the build directory if it is not the default:

SHELL

cmake -B build -DCMAKE_TOOLCHAIN_FILE=$LLVM_WORK/toolchain-ug24.cmake \

-DUG24_TOOLCHAIN=$LLVM_WORK/build-ug24

## 10.  Verification and Testing

### 10.1  Hosted test program (ARM32 and ARM64)

/TMP/HELLO.C

#include <stdio.h>

int main(void) {

printf("Hello from ARM!\n");

return 0;

}

### STEP 10.1.1  ARM32 — compile, inspect, run

SHELL

$LLVM_WORK/toolchain-armhf/bin/clang \

--target=armv7-linux-gnueabihf \

--sysroot=$SYSROOT_ARMHF \

--gcc-toolchain=/usr \

-fuse-ld=lld -static \

-o /tmp/hello-arm /tmp/hello.c

file /tmp/hello-arm

readelf -h /tmp/hello-arm | grep -E 'Class|Machine'

qemu-arm /tmp/hello-arm

EXPECTED OUTPUT

/tmp/hello-arm: ELF 32-bit LSB executable, ARM, EABI5 version 1 (SYSV),

statically linked, not stripped

Class:   ELF32

Machine: ARM

Hello from ARM!

### STEP 10.1.2  ARM64 — compile, inspect, run

SHELL

$LLVM_WORK/toolchain-arm64/bin/clang \

--target=aarch64-linux-gnu \

--sysroot=$SYSROOT_ARM64 \

--gcc-toolchain=/usr \

-fuse-ld=lld -static \

-o /tmp/hello-arm64 /tmp/hello.c

readelf -h /tmp/hello-arm64 | grep -E 'Class|Machine'

qemu-aarch64 /tmp/hello-arm64

EXPECTED OUTPUT

Class:   ELF64

Machine: AArch64

Hello from ARM!

### 10.2  Bare-metal test (arm-none-eabi)

The same hello.c works, because newlib provides printf over semihosting. Compile with clang, link with arm-none-eabi-gcc, which supplies newlib and the semihosting startup files.

SHELL

$LLVM_WORK/toolchain-none-eabi/bin/clang \

--target=arm-none-eabi -mcpu=cortex-m4 -mthumb -O2 \

-isystem /usr/lib/arm-none-eabi/include \

-c /tmp/hello.c -o /tmp/hello-m4.o

arm-none-eabi-gcc -mcpu=cortex-m4 -mthumb /tmp/hello-m4.o \

-o /tmp/hello-m4.elf --specs=rdimon.specs

readelf -h /tmp/hello-m4.elf | grep -E 'Class|Machine'

qemu-system-arm -M mps2-an386 -cpu cortex-m4 -nographic \

-semihosting -kernel /tmp/hello-m4.elf

EXPECTED OUTPUT

Class:   ELF32

Machine: ARM

Hello from ARM!

### STEP 10.2.1  Freestanding link, LLVM only  (requires section 7.2)

Proof that 7.2 worked: a program needing a 64-bit divide, linked with ld.lld and compiler-rt, with no GNU tools involved.

SHELL

cat > /tmp/free.c << 'EOF'

long long divide(long long a, long long b) { return a / b; }

void _start(void) { volatile long long r = divide(1000000LL, 7LL); (void)r; for(;;); }

EOF

RT=$($LLVM_WORK/toolchain-none-eabi/bin/clang --target=armv7em-none-eabi \

-rtlib=compiler-rt -print-libgcc-file-name)

$LLVM_WORK/toolchain-none-eabi/bin/clang --target=armv7em-none-eabi \

-mcpu=cortex-m4 -mthumb -mfpu=fpv4-sp-d16 -mfloat-abi=hard \

-ffreestanding -nostdlib -fuse-ld=lld \

/tmp/free.c $RT -o /tmp/free.elf

file /tmp/free.elf

EXPECTED OUTPUT

/tmp/free.elf: ELF 32-bit LSB executable, ARM, EABI5 version 1 (SYSV),

statically linked, not stripped

### 10.3  uG24 test

Since revision 2.1 the runtime provides <stdio.h>, so the ordinary hello.c compiles and runs. Output still goes through a memory-mapped UART that the simulator models; there is no OS, and no scanf, because there is no input device.

/TMP/HELLO-UG24.C

#include <stdio.h>

int main(void) {

printf("Hello ug24\n");

return 0;

}

SHELL

$LLVM_WORK/build-ug24/bin/clang --target=ug24-unknown-none-eabi -Os \

/tmp/hello-ug24.c -o /tmp/hello-ug24.elf

$LLVM_WORK/build-ug24/bin/llvm-readobj --file-headers /tmp/hello-ug24.elf \

| grep -E 'Format|Machine'

$LLVM_WORK/ug24-sim/ug24sim /tmp/hello-ug24.elf --quiet

EXPECTED OUTPUT

Format: elf32-ug24

Machine: EM_UG24 (0x9240)

Hello ug24

NOTE

USE llvm-readobj, NOT readelf, FOR uG24. System readelf reports 'Machine: <unknown>: 0x9240'. That is expected — EM_UG24 = 0x9240 is a private machine number binutils has never seen. The LLVM tools in build-ug24/bin know it.

### 10.4  Regression suites

SHELL

# uG24: 40 execution cases

$LLVM_WORK/llvm-project/ug24-tests/run-tests.sh

# uG24: the acceptance suite, 15 programs at five optimisation levels

$LLVM_WORK/llvm-project/ug24-tests/suite/run-suite.sh

# uG24: LLVM regression tests

$LLVM_WORK/build-ug24/bin/llvm-lit -s \

$LLVM_WORK/llvm-project/llvm/test/MC/UG24 $LLVM_WORK/llvm-project/llvm/test/CodeGen/UG24

ninja -C $LLVM_WORK/build-ug24 check-llvm-codegen-arm check-llvm-codegen-aarch64

EXPECTED OUTPUT

cases run: 40, failures: 0

PASS

all programs match at all levels

Passed: 12        # uG24 lit

Passed: 4,257     # upstream ARM and AArch64

### 10.5  Helper scripts for uG24 development

| Script | Does |
| --- | --- |
| ug24-tests/suite/run-suite.sh | The acceptance suite: 15 programs at -O0/-O1/-O2/-Os/-O3 |
| ug24-tests/ug24-build.sh f.c | Emits f.ll, f.s, f.o, f.elf and f.dis |
| ug24-tests/ug24-run.sh f.c a b | Compiles, runs, prints the globals named a and b |
| ug24-tests/ug24-asm.sh f.c main | Assembly plus encoded disassembly of one function |

## 11.  Troubleshooting

Every entry below was encountered during a real build.

### cannot open crtbeginS.o  ·  unable to find library -lgcc

A cross link draws from two places. The SYSROOT supplies crt1.o, crti.o, crtn.o and libc; the GCC INSTALLATION supplies crtbeginS.o, crtendS.o and libgcc. Clang does not locate Debian's /usr/lib/gcc-cross layout on its own, so it emits those as bare filenames with no path.

FIX

clang ... --gcc-toolchain=/usr

# confirm it took effect

clang ... -v 2>&1 | grep "Selected GCC installation"

EXPECTED OUTPUT

Selected GCC installation: /usr/lib/gcc-cross/arm-linux-gnueabihf/13

### qemu-arm: Could not open '/lib/ld-linux-armhf.so.3'

The link succeeded — this is a runtime failure. The binary is dynamically linked and qemu is looking for the ARM loader in the x86 root.

FIX — EITHER

clang ... -static

# or

qemu-arm -L /usr/arm-linux-gnueabihf ./hello-arm

### instruction requires: VFP2  (building compiler-rt)

compiler-rt was added to LLVM_ENABLE_RUNTIMES for the bare-metal build, so the sub-build selected -mfloat-abi=soft. Remove it from that build and follow section 7.2 instead.

### undefined symbol: __aeabi_ldivmod  (bare metal)

The bare-metal builtins were not built, or not installed where clang looks. Check with -print-libgcc-file-name (step 7.2.1); if the printed path does not exist, redo section 7.2.

### stdio.h not found

--sysroot is unset or empty. An unset shell variable expands to --sysroot=, which clang silently ignores. Check with ls $SYSROOT_ARMHF/include/stdio.h.

### The wrapper script execs /clang

The heredoc delimiter was unquoted, so $(dirname "$0") was expanded at write time. Recreate it with << 'WRAPPER' — see section 9.1.

### The build is killed part-way through

Out of memory during linking; confirm with dmesg | grep -i oom. Keep -DLLVM_PARALLEL_LINK_JOBS=1 and lower ninja -j below the core count.

### ninja rebuilds hundreds of targets for no reason

A system library update touched a timestamp, so ninja considers llvm-min-tblgen stale and cascades a relink. Harmless, about a minute. The build is done when ninja says 'no work to do'.

### undefined symbol __mulsi3 / __adddf3  (uG24)

Fixed in revision 2.1. long, float, double and long long all work: the helpers are in ug24_builtins.c, ug24_int64.c and ug24_float.c, and %f agrees with a hosted printf digit for digit. A program that prints a float is about 24 KB, because the float runtime is linked on demand and such a program needs it.

## 12.  Build Completion Checklist

| # | Check | Expected |
| --- | --- | --- |
| 1 | Dependencies installed | cmake >= 3.20, ninja >= 1.10 |
| 2 | Environment exported | echo $LLVM_WORK non-empty |
| 3 | Repository cloned | b06ba2a38 |
| 4 | uG24 back end present | 31 files |
| 5 | uG24 workspace present in the clone | ug24-runtime ug24-sim ug24-tests docs |
| 6 | armhf sysroot valid | stdio.h and libc.so present |
| 7 | arm64 sysroot valid | stdio.h and libc.so present |
| 8 | Bare-metal sysroot copied | libc.a, libg.a |
| 9 | Stage 1 built | X86 ARM AArch64 |
| 10 | armhf toolchain built | armv7-unknown-linux-gnueabihf |
| 11 | armhf LLDB present | lldb version 17.0.6 |
| 12 | arm64 toolchain built | aarch64-unknown-linux-gnu |
| 13 | none-eabi toolchain built | arm-none-unknown-eabi |
| 14 | compiler-rt armhf | libclang_rt.builtins-armhf.a |
| 15 | compiler-rt bare metal (optional) | libclang_rt.builtins-armv7em.a |
| 16 | uG24 compiler built | ug24 in --print-targets |
| 17 | ug24-setup.sh and the acceptance suite pass | cases run: 40, failures: 0 · all programs match at all levels |
| 18 | Wrapper scripts work | Hello from ARM! |
| 19 | armhf runs under qemu | Hello from ARM! |
| 20 | arm64 runs under qemu | Hello from ARM! |
| 21 | Bare metal runs under qemu | Hello from ARM! |
| 22 | uG24 runs on the simulator | Hello ug24 |
| 23 | uG24 lit tests | Passed: 12 |
| 24 | Upstream ARM and AArch64 unaffected | 4,257 passed |

### 12.1  Changes from Rev 2.0

| Section | Change |
| --- | --- |
| 1.1, 1.2, 3.2 | The support bundle is gone. The uG24 runtime, simulator, tests and documentation were merged into the repository, so a clone is the whole delivery. The layout and the unpack step were rewritten accordingly. |
| 8.1 | llvm-size and llvm-dwarfdump added to the ninja line. dwarfdump reads unlinked uG24 objects now; before this revision it warned once per relocation. |
| 8.3, 8.4 | ug24-setup.sh is no longer reproduced in full — it ships with the clone. The runtime table gained ug24_int64.c, ug24_float.c, ug24_stdio.c, ug24_stdlib.c and ug24_setjmp.s, and the four standard headers. |
| 10.3, 11 | CORRECTED, twice. Revision 2.0 said hello.c with printf will not compile for uG24, and that long, float and double have no helpers. Neither is true any more: the runtime implements the whole printf family including %f, %e and %g, and every integer width up to 64 bits. |
| 10.4, 10.5 | New: the acceptance suite, fifteen programs at five optimisation levels. The uG24 lit count moved from 7 to 12. |
| 8, 10 | New: -mcpu=ug24-base for a part without the optional multiplier and divider, and __attribute__((interrupt)) for handlers written in C. The interrupt model rests on documented assumptions — see docs/uG24-assumptions.md. |
| 12.2 | Companion document list updated. The requirements answer and the query list are both at Rev 5, and all eleven items that were outstanding at Rev 4 are closed. |
| — | Sections 2, 4, 5, 6, 7 and 9 are unchanged: nothing about the ARM half of the toolchain moved in this revision. |

### 12.2  Companion documents

| File | Contents |
| --- | --- |
| docs/uG24-machine-description.md | Full uG24 instruction encodings, register file, PSW layout |
| docs/uG24-assumptions.md | Every design decision the specification forced, and where to change it |
| docs/uG24_Specification_Queries_ANSWERED.md (Rev 5) | Open questions still with the silicon team. I2, I1 and I3 are the three that would silently invalidate generated code. |
| docs/uG24-build-guide.md · docs/uG24_Toolchain_Requirements_ANSWERED_Rev5.docx | Day-to-day use of the uG24 toolchain; and what became of every item that was outstanding at Rev 4. |