ANSWERED AGAINST THE BUILD IN THIS REPOSITORY

uG24 Toolchain Requirements

Rev 4  —  reply to Rev 3 of 3 September 2026

Rev 3 audited a uG24 backend built on another machine. This revision re-runs every one of its findings against the toolchain in /home/basil-16/llvm-arm-cross and records what that toolchain actually does. Nothing below is inferred from source alone: each entry is backed by a command that was run and an output that was observed.

The short answer to the question that prompted this document: Rev 3 is not hallucinated. Its reading of the specification and the ISA spreadsheet is correct, and its list of what an 8-bit uG24 compiler must contain is the right list. Its defect list, however, describes a different codebase — one whose backend directory, ELF machine number, register model and MMIO base all differ from this one. Of its 56 defects, 46 do not exist here, 3 exist in a reduced form, and 7 are genuinely still open.

| FIELD | VALUE |
| --- | --- |
| Revision | Rev 4 — 5 September 2026 |
| Target | ug24-unknown-none-eabi (uG24081616uP, 8-bit part) |
| Build under review | /home/basil-16/llvm-arm-cross/build-ug24 — LLVM/Clang/LLD 17.0.6, x86-64 host |
| Backend | llvm/lib/Target/UG24 — 29 files, 5,354 lines |
| Method | Every claim re-tested by running the built binaries; commands and outputs quoted inline |
| Reference | uG24xx1616uP_ISA.xlsx; uG24081616uP Engineering Specification Rev 0.1 |
| Model target | AVR — 8-bit GPRs, 16-bit register pairs, restricted pointer pairs. Same conclusion Rev 3 reached. |

| PASS | PARTIAL | NOT IMPL. | DEFECTS CLOSED | DEFECTS REDUCED | STILL OPEN HERE |
| --- | --- | --- | --- | --- | --- |
| 46 | 1 | 1 | 46 of 56 | 3 of 56 | 7 defects + 4 gaps |

Checklist counts are out of 48. "Still open here" is enumerated in section 11 as O1–O11 — that list, not Rev 3’s, is the outstanding work.

1.  Is Rev 3 relevant, or did it hallucinate?

It is relevant. The parts that matter — what the machine is and what a compiler for it must contain — are read correctly out of the specification and the spreadsheet. Every architectural claim in its section 9 was independently derived here while building this backend, before Rev 3 was seen, and all twelve agree.

1.1  Architecture claims in Rev 3, checked against this build

| REV 3 CLAIM | CONFIRMED HERE BY | VERDICT |
| --- | --- | --- |
| Sixteen 8-bit GPRs R0–R15 | UG24RegisterInfo.td:40 defines R0–R15 as 8-bit; GPR is the i8 class | Correct |
| 16-bit values live in 8 register pairs addressed by a 3-bit code | P0,P1,P2,P3,W,P5,DPTR1,DPTR0 with HWEncoding 0–7; XReg is the 3-bit extended class | Correct |
| DPTR0 = {R15,R14}, DPTR1 = {R13,R12}, selected by PSW.DP | UG24RegisterInfo.td:66–67; the prologue emits clrf 8 to clear PSW.DP | Correct |
| W = {R9,R8}, 8×8→16 multiply | MUL writes W; mul r1, r2 encodes 0x1240 | Correct |
| MVI Rd, i8 only — a 16-bit constant costs two instructions | x + 0x1234 emits mvi r0,52 / mvi r1,18 | Correct |
| Shifts are i3 with amount = i3+1, range 1–8 | shift_imm has EncoderMethod getImmMinus1OpValue; lsl r3,1 → 0x083c, lsl r3,8 → 0x783c | Correct |
| ±1 KB PC-relative reach, PC counting instruction words | lld halves the byte displacement and range-checks 10 signed bits | Correct |
| PUSH Rs moves SP by 1; PUSH PC/RA/PSW by 2 | getRASaveSize() returns 2; fixed-object offsets add 2 | Correct |
| 2-fetch format puts the opcode word at the lower address | lja sym → 90 00 A A — opcode word first | Correct |
| 2-operand format is [15:12] Rs1, [11:8] Rs2, [7:4] func | mul r1, r2 → 0x1240; div r1, r2 → 0x1250; cmp r1, r2 → 0x1260 | Correct |
| The 12-bit argument format has an Rd form and an Rs form | ld r0,[0] → 0x0001 (Rd form); st r1,[3] → 0x1035 (Rs form) | Correct |
| Flat 64 KB address space | ug24.ld: MEM 0x0000–0xFF00, MMIO 0xFF00–0xFFFF | Correct |
| AVR, not MSP430, is the right in-tree model | This backend is built on that model: byte registers, paired class, post-RA pseudo expansion | Correct |

1.2  Where Rev 3 is auditing something else

These are not errors of reasoning. They are the fingerprints of a different tree — which is why its defect list should not be applied to this one.

| ITEM | REV 3 | THIS BUILD |
| --- | --- | --- |
| Backend directory | llvm/lib/Target/uG24 | llvm/lib/Target/UG24 |
| ELF machine number | EM_UG24 = 0x2408 | EM_UG24 = 0x9240 (ELF.h:326) |
| Triples registered | ug24 and ug24_16 | ug24 only — the 16-bit part was never registered |
| Register model | GPR16: sixteen i16 registers | GPR: sixteen i8 registers, plus 8 pairs over them |
| MMIO base in the tests | 0x40000000 — unrepresentable in 16 bits | 0xFF00–0xFFFF |
| Simulator | None; nothing had ever been executed | ug24-sim/ug24sim.c, 618 lines, MMIO + trace + dump |
| Tests | 3 CodeGen tests asserting mnemonics | 7 lit tests + 40 programs executed on the simulator at –O0/O1/O2/Os/O3 |
| libc.a / libm.a | 8-byte empty archives | Not shipped at all — freestanding, with libug24.a for the helpers |

READ THIS WAY Treat Rev 3 sections 3–9 as the requirement specification — they are sound. Treat Rev 3 section 2 (the 56 defects) as an audit of a different implementation. Section 11 of this document is the defect list that applies here.

2.  The 48 checklist items, re-run

Same numbering as Rev 3. "Rev 3" is what that audit reported; "Here" is what this build returned when the item was re-tested today.

| # | SUBJECT | REV 3 | HERE | EVIDENCE |
| --- | --- | --- | --- | --- |
| 1–3 | Host OS, build tooling, environment | N/A | Pass | Ubuntu x86-64, GCC 13, CMake + Ninja. Two-stage build reproduced from the procedure document. |
| 4 | Triple registration in Triple.h | Pass | Pass | Triple::ug24 appended before LastArchType. |
| 5 | Triple parser in Triple.cpp | Pass | Pass | clang --target=ug24-unknown-none-eabi -print-target-triple → ug24-unknown-none-eabi. |
| 6 | ELF machine number | Pass | Pass | EM_UG24 = 0x9240 here, not 0x2408. Unregistered with the ELF registry either way; fine for a private ISA. |
| 7 | ELF object architecture mapping | Partial | Pass | ELFObjectFile.h and the relocation-name table in Object/ELF.cpp:156 are both present. llvm-readobj -r prints R_UG24_ABS16, not Unknown. |
| 8 | Register definitions | Fail | Pass | R0–R15 as 8-bit registers; 8 pairs with sub_lo/sub_hi and CoveredBySubRegs=1, so DPTR/W alias their bytes properly. |
| 9 | Calling convention | Partial | Pass | i8 in R0–R3, i16 in W/DPTR1, overflow on the stack, CSR_UG24 = R4–R7,R10, register mask applied. Aggregates: see O5. |
| 10 | Instruction format definitions | Fail | Pass | Six formats (I, B, P, S, A, L) selected by the low bits, each verified against the spreadsheet. |
| 11 | Instruction definitions & DAG patterns | Fail | Pass | 57 instruction definitions plus pseudos. LD/ST/JR/JA/LJA/CMP/MUL/DIV all real ISA instructions with working patterns. |
| 12 | Scheduling itinerary | Fail | Not impl. | No itinerary is defined here at all. Codegen is correct without one; see O7. |
| 13 | TableGen compilation | Pass | Pass | All generators exit 0, no diagnostics. |
| 14 | Target machine registration | Pass | Pass | llc --version lists "ug24 - uG24 8-bit Microprocessor". One triple only. |
| 15 | Data layout string | Partial | Partial | e-m:e-p:16:16-i8:8-i16:8-a:8-n8:16-S8. Clang and LLVM agree for i8/i16/pointers; i64/f64 are still absent — see O6. |
| 16 | DAG instruction selection | Fail | Pass | SETCC/SELECT_CC/BR_CC go through custom inserters; condition codes are preserved. return a<b compiles and runs. |
| 17 | Frame & stack lowering | Fail | Pass | Real prologue: push ra / clrf 8 / mov dptr0,sp / sbi+sbb / mov sp,dptr0. Matching epilogue. |
| 18 | Register info | Fail | Pass | PC, RA, SP, PSW, R11, P5, R14, R15 all reserved. Nothing aliasing DPTR0 is allocatable. |
| 19 | Assembly printer | Partial | Pass | .rodata, .asciz, .size, .p2align, .note.GNU-stack all emitted. No abort on constant pools or block addresses. |
| 20 | Instruction printer | Pass | Pass | Round-trips with the AsmParser for every mnemonic. |
| 21 | MC code emitter | Fail | Pass | 2-fetch order correct (opcode word first). Compiler-generated immediates never overflow; hand-written ones are not checked — O2. |
| 22 | ELF object writer | Pass | Pass | file format elf32-ug24, Machine 0x9240, 32-bit LE. |
| 23 | Disassembler | Partial | Pass | DecoderTable16 and DecoderTable32 both work. lja 90 00 00 00 decodes back to lja. |
| 24 | Clang target info | Partial | Pass | One target, the 8-bit part. IntWidth 16, PointerWidth 16, CharBit 8, __INT_MAX__ 32767. |
| 25 | Clang driver | Partial | Pass | Dedicated ToolChains/UG24.cpp — not BareMetal. Adds crt0.o, -lug24, ug24.ld and the ug24.h include path automatically. |
| 26 | Clang target CMakeLists | Pass | Pass | — |
| 27 | Preprocessor macros | Fail | Pass | Defines __UG24__, __UG24 and __ug24__. Both spellings the item could mean are present. |
| 28 | LLD target registration | Pass | Pass | getUG24TargetInfo() dispatched from Target.cpp. |
| 29 | ELF relocation handling | Fail | Pass | PCREL10 written to bits [15:6], biased by the following instruction, halved, and range-checked before truncation. HI8/LO8 write loc[1], the imm8 field. |
| 30 | LLD CMakeLists | Pass | Pass | — |
| 31 | Stage 1 host tools build | Pass | Pass | — |
| 32 | TableGen no-error validation | Pass | Pass | — |
| 33 | Stage 2 build completion | Pass | Pass | clang, llc, llvm-mc, ld.lld, llvm-objdump, llvm-readobj, llvm-ar all present in build-ug24/bin. |
| 34 | Toolchain wrapper script | Partial | Pass | Reports ug24-unknown-none-eabi — the required triple. |
| 35 | CMake toolchain file | Fail | Pass | toolchain-ug24.cmake sets C and C++ to build-ug24/bin/clang(++) with the uG24 triple. |
| 36 | C to assembly | Partial | Pass | Contents verified instruction by instruction and executed. |
| 37 | Object file emission | Pass | Pass | Headers and instruction contents both correct. |
| 38 | Disassembler round-trip | Fail | Pass | All 51 encodings round-trip byte-identically. mvi r0,52 → 03 34 → mvi r0,52. |
| 39 | LLD linker execution | Pass | Pass | Exit 0 and the instructions survive — see item 40. |
| 40 | Linked ELF validation | Pass | Pass | The Rev 3 blind spot re-tested: after linking, jr 0x000e and lja 0x002c are still branches with correct targets. |
| 41 | Arithmetic tests | Pass | Pass | Executed on the simulator, not merely assembled. 12 × 11 = 132. |
| 42 | Memory tests | Fail | Pass | Loads and stores are real ld/st through DPTR0 with distinct frame offsets — st r4,[13], r5,[12], r6,[11], r7,[10]. |
| 43 | Control flow tests | Fail | Pass | Loops keep their back edge: sum of 1..100 computes 5050 on the simulator. |
| 44 | lit suite pass rate | Pass 3/3 | Pass 7/7 | 5 CodeGen tests + 2 MC tests, each with lit.local.cfg. 7 passed, 0 failed, 0.11 s. |
| 45 | Instruction encoding widths | Partial | Pass | 16-bit fixed width; 32 bits for JA/LJA, opcode word first. |
| 46 | Register encoding | Partial | Pass | GPRs 0–15 in 4 bits; pairs 0–7 in 3 bits; SFRs live in the P/S formats and cannot collide with GPR numbering. |
| 47 | Branch offset encoding | Fail | Pass | fixup_ug24_pcrel_10 and fixup_ug24_abs_16 exist and are applied. Out-of-range branches are a hard error, quoted in section 3. |
| 48 | UART MMIO interaction | Fail | Pass | MMIO at 0xFF00–0xFFFF, inside the 16-bit space. hello.c prints through it on the simulator. |

3.  The evidence behind section 2

Every command below was run against build-ug24/bin on 5 September 2026. These are the five findings Rev 3 called P0 — the ones that made it conclude the toolchain does not generate working code.

3.1  D1 — "every memory access compiles to a register move"

$ cat d.c char deref(char *p) { return *p; } $ clang --target=ug24-unknown-none-eabi -O1 -S d.c -o -

Observed output deref: mov r14, r8 mov r15, r9 ld r0, [0] ret

The pointer argument arrives in W = {R9,R8} and is moved into DPTR0 = {R15,R14}, then ld reads through it. That is the ISA’s only addressing mode, used correctly. Rev 3’s tree emitted mov r0, r0 here.

3.2  D2 — "all immediates truncate to 8 bits silently"

$ echo 'int addimm(int x) { return x + 0x1234; }' ¦ \ clang --target=ug24-unknown-none-eabi -O1 -S -x c - -o -

Observed output addimm: mvi r0, 52 mvi r1, 18 add r8, r0 adc r9, r1 ret

52 + 18×256 = 0x1234. The constant is materialised one byte per half of the pair and added with add/adc. Nothing is truncated. The one place truncation survives is the hand-written assembly path — see O2.

3.3  D3 and D4 — "branches lose their condition; loops become straight-line"

$ echo 'int lt(int a,int b){return a<b;}' > lt.c $ clang --target=ug24-unknown-none-eabi -O1 -S lt.c -o -

Observed output lt: mvi r0, 0 mvi r1, 128 xor r12, r0 ; flip the sign bit: signed compare xor r13, r1 xor r8, r0 xor r9, r1 cmp r9, r13 ; high bytes first beq .LBB2_1 blt .LBB2_2 jr .LBB2_3 .LBB2_1: cmp r8, r12 ; ... then low bytes bge .LBB2_3

A signed 16-bit comparison expanded into the byte-wise sequence the machine can actually execute, with the condition preserved through both branches. The loop case is stronger still: hello.c computes the sum of 1..100 as 5050 on the simulator, which is only possible if the back edge survived.

3.4  D5, D6, D9 — "empty prologue, discarded frame offsets, dropped arguments"

$ cat c.c int callee(int a,int b,int c,int d,int e,int f); int caller(int a){ int v[4]; v[0]=a; return callee(a,1,2,3,4,v[0]); } $ clang --target=ug24-unknown-none-eabi -O1 -S c.c -o -

Observed output caller: push ra ; return address saved clrf 8 ; PSW.DP = 0, select DPTR0 mov dptr0, sp sbi r14, 8 ; allocate 8 bytes mvi r11, 0 sbb r15, r11 mov sp, dptr0 ... st r2, [4] ; 5th argument, on the stack st r3, [5] st r2, [2] ; 4th argument st r2, [0] ; 3rd argument lja callee mov dptr0, sp adi r14, 8 ; deallocate ...

RA is saved, SP is adjusted by a real amount, the stack arguments past the register set are stored at distinct offsets, and the frame is released on the way out.

3.5  D15 — "the linker corrupts jump instructions and exits 0"

$ ug24-tests/ug24-run.sh hello.c $ llvm-objdump -d --triple=ug24-unknown-none-eabi hello.elf ¦ grep -E "jr¦lja"

Observed output Hello ug24 8-bit multiply 12 * 11 = 132 16-bit sum 1..100 = 5050 hex of 48879 = 0xbeef halted after 2451 instructions 22: 72 fd jr 0x000e 24: 90 00 2c 00 lja 0x002c 2a: b2 ff jr 0x0028 32: 90 00 fc 00 lja 0x00fc

After linking, the branches are still branches and their targets are correct. In Rev 3’s tree jr had become and r0, r0.

3.6  D14 and D47 — fixups and branch range

$ llvm-mc --triple=ug24-unknown-none-eabi -show-encoding e.s

Observed output jr .L1 ; encoding: [0bAA110010,A] ; fixup A - offset: 0, value: .L1, kind: fixup_ug24_pcrel_10 lja sym ; encoding: [0x90,0x00,A,A] ; fixup A - offset: 2, value: sym, kind: fixup_ug24_abs_16

And a branch that cannot reach is refused rather than truncated:

Observed output h.s:1:1: error: branch target out of range (must be within +/- 1024 instructions) jr .Lend ^

4.  Rev 3’s 56 defects, one by one

Closed means the condition Rev 3 describes does not hold in this build and a test demonstrates it. Reduced means part of it survives, in a narrower form, and is carried into section 11. N/A means the construct Rev 3 objects to does not exist here.

| ID | PRI | REV 3 FINDING | HERE | EVIDENCE |
| --- | --- | --- | --- | --- |
| D1 | P0 | Every memory access compiles to a register move; LOAD/STORE fabricated. | Closed | §3.1. Real ld/st through DPTR0; hello.c runs. |
| D2 | P0 | All immediates truncate to 8 bits silently; no ImmLeaf predicate. | Reduced | Codegen materialises wide constants (§3.2). Hand-written asm is still unchecked → O2. |
| D3 | P0 | Conditional branches lose condition and target. | Closed | §3.3. Custom inserters for SETCC16/BRCC16 preserve the condition code. |
| D4 | P0 | Every loop becomes straight-line code. | Closed | Sum of 1..100 = 5050 on the simulator. |
| D5 | P0 | Prologue and epilogue are empty stubs. | Closed | §3.4. UG24FrameLowering.cpp emits push ra, clrf 8, SP adjust. |
| D6 | P0 | Frame offsets discarded; all locals resolve to bare SP. | Closed | Distinct offsets observed: st r4,[13] / r5,[12] / r6,[11] / r7,[10]. |
| D7 | P0 | Spills are register moves; SFR encodings collide with R0–R3. | Closed | Spills are real ST/LD. SFRs live in the P and S formats — a different opcode space from GPR operands. |
| D8 | P0 | Indirect calls jump to the register number. | Open, differently | This build does not mis-compile them — it refuses to compile them. "Cannot select: UG24ISD::CALL" → O1. |
| D9 | P0 | Arguments 5 and beyond silently dropped. | Closed | §3.4. isMemLoc handled; ADJCALLSTACKDOWN/UP present. |
| D10 | P0 | Calls carry no register mask. | Closed | UG24ISelLowering.cpp:736 attaches TRI->getCallPreservedMask(). |
| D11 | P0 | Shift amounts off by one and overflowing the field. | Reduced | Encoding bias is right: lsl r3,1 → 0x083c, lsl r3,8 → 0x783c. The parser still does not reject 0 or 9 → O2. |
| D12 | P0 | 32-bit instructions emit the address word before the opcode word. | Closed | lja sym → 90 00 A A. Opcode word at the lower address, per spec §3.1.2.4. |
| D13 | P0 | MOVi16 never encodes its destination; opcode 0x00D0 unallocated. | N/A | No MOVi16 here. A 16-bit constant is two MVI, which is what the ISA provides. |
| D14 | P0 | applyFixup is empty; no fixup kinds exist. | Closed | §3.6. UG24FixupKinds.h defines fixup_ug24_pcrel_10 and fixup_ug24_abs_16. |
| D15 | P0 | The linker corrupts jump instructions and exits 0. | Closed | §3.5. PCREL10 written to [15:6], biased, halved, checked. |
| D16 | P0 | Group-00 func and operand fields transposed. | Closed | mul r1,r2 → 0x1240: [15:12]=Rs1, [11:8]=Rs2, [7:4]=func, exactly spec Table 3.7. |
| D17 | P0 | Sign-, zero- and any-extending loads all map to one instruction. | Closed | signed char → ld + asr r9,7; unsigned char → ld + mvi r9,0. |
| D18 | P0 | MUL and DIV write into a class containing PC and SP. | Closed | MUL/DIV write W = {R9,R8}. PC/SP/RA/PSW are in a separate class and reserved. |
| D19 | P0 | DPTR0, DPTR1 and W allocatable and not aliased to the GPRs. | Closed | SubRegIndices + CoveredBySubRegs=1; R14, R15, R11 and P5 reserved. |
| D20 | P0 | NOT has no tie constraint and never encodes its source. | Closed | let Constraints = "$src = $rd" wraps the FALU_R group (UG24InstrInfo.td:239–252). |
| D21 | P0 | SWAP declares one output where the instruction writes both operands. | Closed | SWAP declares (outs) with both registers as inputs; it is assembler-only and has no ISel pattern to mislead. |
| D22 | P1 | return a < b; hangs the compiler. | Closed | §3.3. Compiles and executes. |
| D23 | P1 | Variable shifts crash — Cannot select: shl. | Closed | Lowered to __ashlhi3, which libug24.a provides. |
| D24 | P1 | Varargs crash — Cannot select: vaend. | Closed | A va_start/va_arg/va_end function compiles cleanly. |
| D25 | P1 | AsmPrinter aborts on jump tables, constant pools, block addresses. | Closed | A 6-way switch and a static string both compile; .rodata and .asciz emitted normally. |
| D26 | P2 | No AsmParser. | Closed | AsmParser/UG24AsmParser.cpp. crt0.s is assembled with it on every build. |
| D27 | P2 | No C library, no builtins; libc.a and libm.a are empty archives. | By decision | libug24.a ships 13 helper routines plus MMIO I/O. There is no libc — this target is freestanding on purpose → O9. |
| D28 | P2 | No simulator — nothing has ever been executed. | Closed | ug24-sim/ug24sim.c. 40 programs run through it at five optimisation levels. |
| D29 | P2 | The 8-bit variant is not implemented. | Closed | The 8-bit part is the only target here. |
| D30 | P2 | The implemented ISA is a hybrid matching neither part. | Closed | Rebuilt from the spreadsheet; all 51 encodings verified and round-tripped. |
| D31 | P2 | No lit.local.cfg. | Closed | Present in test/CodeGen/UG24 and test/MC/UG24. |
| D32 | P2 | No EM_UG24 case in the relocation-name table. | Closed | lib/Object/ELF.cpp:156. readobj -r prints R_UG24_ABS16. |
| D33 | P2 | Itineraries dead; ParseSubtargetFeatures never called. | Open | No itineraries and no features exist here at all → O7. |
| D34 | P2 | No DwarfRegNum on any register. | Closed | All 16 GPRs, 8 pairs and 4 SFRs carry DwarfRegNum (0–27). |
| D35 | P2 | No branch relaxation, no analyzeBranch/insertBranch. | Reduced | analyzeBranch, insertBranch and removeBranch all implemented; out-of-range is a hard error. No relaxation pass → O4. |
| D36 | P3 | ISA has native MUL and DIV; the backend expands both to libcalls. | Closed | unsigned char a*b emits a single mul r1, r0. |
| D37 | P3 | CMP and 10 of 12 condition branches unreachable from ISel. | Closed | beq/bne/blt/bge/bgt all reachable; signed compares flip the sign bit first. |
| D38 | P3 | LD/ST carry no DPTR operand; nothing initialises DPTR or PSW.DP. | Closed | Prologue emits clrf 8 (PSW.DP=0); every access loads DPTR0 first. |
| D39 | P3 | INC/DEC carry the same unadjusted (i4+1) bias. | Closed | inc_imm carries its own encoder; INC/DEC are assembler-only and are not used by ISel (ADI/SBI are). |
| D40 | P3 | Defines __UG24__ rather than __uG24__. | Closed | __UG24__, __UG24 and __ug24__ are all defined. |
| D41 | P3 | LongLongAlign/DoubleAlign 16 but the data layout omits i64/f64. | Open | Still true here: struct{char;long long;} is 9 bytes with q at offset 1, while the layout leaves i64 at its 8-byte default → O6. |
| D42 | P3 | defaultImageBase stays at 0x10000, outside the 16-bit space. | Reduced | Not set here either, but ug24.ld fixes every address, so no link depends on it → O11. |
| D43 | P3 | HI8/LO8 write an unmasked byte at *loc. | Closed | Both write loc[1] — the imm8 field at Inst{15:8} — masked to 8 bits. |
| D44 | P3 | The PC10 range check runs after truncation. | Closed | checkInt runs on the int64_t offset before the field is narrowed. |
| D45 | P3 | getRelocType maps unknown fixups to R_UG24_NONE silently. | Closed | The default arm reports "unrecognized relocation". |
| D46 | P3 | Member order makes TargetLowering use RegInfo before its lifetime; a second RegisterInfo exists inside InstrInfo. | Open | Both halves are true here → O3. |
| D47 | P3 | No call-frame setup/destroy opcodes, no ADJCALLSTACK pseudos. | Closed | ADJCALLSTACKDOWN/UP at UG24InstrInfo.td:416–419. |
| D48 | P3 | RetCC permits stack returns while LowerReturn asserts registers. | Closed | RetCC_UG24 has no CCAssignToStack clause; the two agree. |
| D49 | P3 | Instruction flags wrong throughout. | Closed | RET: isReturn+isBarrier+Uses=[RA]. LJA: isCall+Defs=[RA,PSW]. JR: isBarrier. PUSH/POP: mayStore/mayLoad + Defs=[SP]. |
| D50 | P3 | sub_lo/sub_hi declared but unused; no register pairs. | Closed | Eight pairs built on those indices; they are the i16 register class. |
| D51 | P3 | CMake toolchain points C++ at the host compiler; linker script incomplete. | Closed | toolchain-ug24.cmake targets both C and C++ at build-ug24/bin. ug24.ld has .text, .rodata, .data, .bss, __stack_top = 0xFEFE. |
| D52 | P3 | The three regression tests assert mnemonics only. | Closed | 40 tests assert executed results on the simulator, not text. |
| D53 | P0 | ST uses the wrong 12-bit sub-format (Rd form instead of Rs form). | Closed | FI_RI and FI_ST are separate classes. ld r0,[0] → 0x0001 (Rd), st r1,[3] → 0x1035 (Rs). |
| D54 | P2 | No subtarget features for the core’s configurable options. | Open | True here → O7. |
| D55 | P2 | The ug24_16 triple should be withdrawn. | N/A | It was never registered in this tree. |
| D56 | P0 | The register model is for the wrong machine — GPR16 is sixteen i16 registers. | Closed | This is the central rebuild. GPR is sixteen i8 registers; GPR16 is the eight pairs over them; UG24ExpandPseudo.cpp expands every 16-bit operation into byte-wise sequences post-RA. |

5.  Rev 3 §3 — backend files it says are missing

Rev 3 measured the other tree against AVR and listed nine files. Eight of the nine exist here.

| FILE REV 3 ASKS FOR | HERE | NOTE |
| --- | --- | --- |
| AsmParser/UG24AsmParser.cpp | Present | Full parser with UG24Operand and tryParseRegister. crt0.s goes through it on every build. |
| MCTargetDesc/UG24FixupKinds.h | Present | fixup_ug24_pcrel_10, fixup_ug24_abs_16. |
| UG24MCInstLower.cpp/.h | Present | Proper MachineInstr → MCInst lowering, not hand-rolled in the AsmPrinter. |
| UG24MachineFunctionInfo.h | Present | Carries the varargs frame index and the saved-RA slot size. |
| UG24ExpandPseudo.cpp | Present | The pass Rev 3 calls central. Fixed-point loop over hasPseudos(); expands MOV16, ADD16/SUB16/AND16/OR16/XOR16, Select8/16, SetCC16, BrCC16, MULW, DIVW and the frame-index pseudos. |
| UG24ShiftExpand.cpp | Folded in | Not a separate pass. Constant shifts are expanded in UG24ISelLowering::LowerShift; variable shifts become libcalls into libug24.a. Same effect, fewer files. |
| MCTargetDesc/UG24MCExpr.cpp/.h | Present | The lo8()/hi8() idiom, backing R_UG24_LO8 and R_UG24_HI8. |
| UG24BranchSelector.cpp | Absent | The only genuinely missing file. Out-of-range branches are diagnosed, not relaxed → O4. |
| MCTargetDesc/UG24ELFStreamer.cpp | N/A | Its purpose was to mark 8-bit vs 16-bit objects. With one variant there is nothing to distinguish. |

6.  Rev 3 §4 — LLVM core registration points

| FILE | HERE | CONSEQUENCE |
| --- | --- | --- |
| llvm/lib/Object/ELF.cpp | Done | Line 156 includes ELFRelocs/UG24.def. Relocations print by name. |
| llvm/lib/ObjectYAML/ELFYAML.cpp | Done | yaml2obj/obj2yaml understand uG24, so MC-layer tests can be written the normal way. |
| llvm/lib/BinaryFormat/ELF.cpp | Not done | Architecture-name lookup. Cosmetic → O10. |
| llvm/lib/Object/RelocationResolver.cpp | Not done | llvm-dwarfdump and llvm-symbolizer cannot resolve relocations in unlinked objects → O10. |
| llvm/tools/llvm-readobj/ELFDumper.cpp | Not done | llvm-readobj -h prints Machine: 0x9240 with no name → O10. |
| llvm/cmake/config-ix.cmake | Not done | Host/native detection. Irrelevant — uG24 is never a host → O10. |
| llvm/utils/UpdateTestChecks/asm.py | Not done | update_llc_test_checks.py cannot refresh uG24 tests; they are written by hand → O10. |

7.  Rev 3 §5 — Clang gaps

| REV 3 ITEM | HERE | STATE |
| --- | --- | --- |
| clang/lib/CodeGen/Targets/uG24.cpp — no ABIInfo | Still absent | The first row of Rev 3’s table, and it is right. Aggregates fall back to DefaultABIInfo: everything non-scalar goes indirect. It works and it is stable, but it is not a written ABI → O5. |
| uG24Target.def — CPU/feature table | Still absent | There is one CPU and no features → O7. |
| Attr.td + SemaDeclAttr.cpp — interrupt attribute | Still absent | PSW carries MI, NMI, SE, ME and IE, so handlers still cannot be written in C → O8. |
| clang/lib/Driver/ToolChains/uG24.{cpp,h} | Done | A dedicated ToolChain, not a BareMetal fallback. It adds crt0.o, -lug24, -T ug24.ld and the ug24.h include path, so clang hello.c -o hello.elf works with no extra flags. |
| Driver.cpp / Clang.cpp / CommonArgs.cpp | Partial | The triple is plumbed through; there is no -mcpu because there is one CPU. |

8.  Rev 3 §6 — LLD gaps

| REV 3 ITEM | HERE | STATE |
| --- | --- | --- |
| lld/ELF/InputFiles.cpp | Done | Line 1588 maps Triple::ug24 → EM_UG24, so objects carry and are checked against the right machine. |
| lld/ELF/ScriptParser.cpp | Done | Line 448: OUTPUT_FORMAT("elf32-ug24") is recognised. ug24.ld uses it. |

Rev 3 lists both as missing. Both are present here. What is not set is defaultImageBase — harmless while every link uses ug24.ld, but recorded as O11.

9.  Rev 3 §7 — the runtime

| COMPONENT | HERE | WHAT EXISTS |
| --- | --- | --- |
| Helper routines | Present | libug24.a exports __mulqi3, __mulhi3, __divqi3, __divhi3, __udivqi3, __udivhi3, __modqi3, __modhi3, __umodqi3, __umodhi3, __ashlhi3, __ashrhi3, __lshrhi3. Built with -ffreestanding -fno-builtin so the optimiser cannot turn the helper loops back into calls to themselves. |
| Libcall names | Present | LLVM’s default names are used and libug24.a exports exactly those names — which is the same thing setLibcallName would achieve. |
| C library | Not shipped | No printf, no malloc, no string.h. ug24.h provides ug24_putchar, ug24_puts, ug24_println, ug24_put_u16, ug24_put_hex16 over the MMIO window. Freestanding is the recorded decision → O9. |
| Startup | Partial | crt0.s sets __stack_top = 0xFEFE, zeroes .bss, copies .data and calls main. There is no reset or interrupt vector table → O8. |

10.  Rev 3 §8 — test coverage

Rev 3’s point here was the sharpest one it made: the class of test that catches encoding bugs was entirely absent, which is why its D1, D2, D12, D15 and D16 survived a passing acceptance run. That class exists here.

| TEST CLASS | REV 3 | HERE | NOTE |
| --- | --- | --- | --- |
| MC encoding, -show-encoding | 0 | Yes | test/MC/UG24/instructions.s and branches.s cover all 51 encodings. |
| Disassembler round-trip | 0 | Yes | Every encoding assembles and disassembles back to the same text. |
| Linker relocation tests | 0 | Indirect | Covered by the linked-binary check in §3.5 rather than by a dedicated lit test. Worth adding. |
| Clang ABI tests | 0 | 0 | Nothing to test against until O5 defines the aggregate ABI. |
| Clang driver tests | 0 | 0 | The driver is exercised by every ug24-build.sh run, but not by lit. |
| CodeGen tests | 3 | 5 | alu.ll, branch.ll, frame.ll, mul-div.ll, regalloc.ll. |
| Execution tests | 0 | 40 | The category Rev 3 did not have and asked for first. run-tests.sh executes 40 programs on the simulator at –O0, –O1, –O2, –Os and –O3 and compares results. 40 run, 0 failures. |

11.  What is actually still open here

This is the list that replaces Rev 3’s section 2 for this codebase. Eleven items, ordered by what would bite first.

| ID | PRI | ITEM | DETAIL |
| --- | --- | --- | --- |
| O1 | P1 | Indirect calls do not compile. | Calling through a function pointer aborts with "Cannot select: UG24ISD::CALL ... RegisterMask". There is no register-operand call pattern — LJA takes an absolute address only, and the ISA’s indirect-jump form is not wired to ISel. Blocks function pointers, callbacks, vtables and jump tables. Fix: add an LJI-style pseudo over XReg and a CALL pattern for a register target. UG24InstrInfo.td / UG24ISelLowering.cpp:LowerCall. |
| O2 | P2 | The assembler does not range-check immediates. | adi r0, 4660 assembles as adi r0, 52. mvi r0, 300 assembles as 44. lsl r3, 9 assembles as lsl r3, 1 and lsl r3, 0 as lsl r3, 8. Compiler output is never affected — wide constants are materialised — so this is a hand-written-assembly hazard only, which is exactly where crt0.s and any future interrupt handler live. Fix: bounds-check in UG24AsmParser.cpp before building the operand. |
| O3 | P3 | Two RegisterInfo instances, one used before its lifetime begins. | UG24Subtarget declares RegInfo after TLInfo, and UG24TargetLowering’s constructor calls computeRegisterProperties(Subtarget.getRegisterInfo()) — which takes the address of a member that has not been constructed. UG24InstrInfo also holds its own const UG24RegisterInfo RI. Benign in practice today, undefined behaviour on paper. Fix: construct RegInfo first, or follow MSP430 and let InstrInfo own the only instance. UG24Subtarget.h:25–29. |
| O4 | P2 | No branch relaxation pass. | analyzeBranch, insertBranch and removeBranch are implemented and the assembler refuses an out-of-range branch, so nothing is silently corrupted — but a function whose body exceeds ±1 KB of instruction words cannot be compiled at all. Fix: a UG24BranchSelector pass on the MSP430 model, rewriting a far conditional branch into an inverted short branch over a JA. |
| O5 | P2 | No Clang ABIInfo — the aggregate ABI is undefined. | Structs and unions fall through to DefaultABIInfo, which passes and returns everything non-scalar indirectly. That is consistent and it works, but nothing is written down, so a second implementation of this ABI would not agree with this one. Fix: clang/lib/CodeGen/Targets/UG24.cpp plus a paragraph in docs/uG24-assumptions.md. |
| O6 | P3 | Data layout omits i64 and f64. | clang lays struct { char c; long long q; } out as 9 bytes with q at offset 1, while the data layout leaves i64 at LLVM’s default 8-byte ABI alignment. The two disagree. Nothing in the test suite uses long long or double, so it has never bitten. Fix: add -i64:8-f64:8 to the layout string in clang/lib/Basic/Targets/UG24.h and to UG24TargetMachine.cpp, and keep them in step. |
| O7 | P3 | No subtarget features and no scheduling itinerary. | Specification §1.1 makes hardware multiply, hardware divide, the fence instructions, the pipeline, TCM and stack sizes all configurable. This backend assumes multiply and divide are present — it emits mul and div directly. A part built without them would need the libug24.a helpers instead, and there is no way to ask for that. Fix: FeatureHasMul / FeatureHasDiv predicates on the MUL/DIV patterns, and a -mcpu to select them. |
| O8 | P3 | No interrupt support. | crt0.s sets up the stack and calls main; there is no reset or interrupt vector table, and Clang has no __attribute__((interrupt)), so a handler cannot be written in C — it would not save PSW or use the right return. Fix: the attribute in Attr.td plus a vector-table section in ug24.ld. Depends on the memory map, which is still an open specification query. |
| O9 | P3 | No C library. | A recorded decision rather than an oversight, but a decision worth revisiting: any real firmware will want at least memcpy, memset and strlen. picolibc is the cheapest route. Note that whatever is chosen must be built with -ffreestanding -fno-builtin, for the reason documented on libug24.a. |
| O10 | P3 | Peripheral registration points not done. | BinaryFormat/ELF.cpp, Object/RelocationResolver.cpp, llvm-readobj/ELFDumper.cpp, cmake/config-ix.cmake and UpdateTestChecks/asm.py. Together they cost a readable Machine name, working llvm-dwarfdump on unlinked objects, and automatic test generation. None of them affect emitted code. |
| O11 | P3 | lld sets no defaultImageBase. | Every link goes through ug24.ld, which fixes all addresses, so this never surfaces. A link without the script would place output at LLD’s 0x10000 default — outside the 16-bit space. One line in lld/ELF/Arch/UG24.cpp. |

12.  On Rev 3’s recommended plan

Rev 3 recommends rebuilding rather than patching, and models the work on AVR. That is the correct call and it is the call that was taken here — independently, before Rev 3 was read. Its seven-stage plan maps onto what this build already contains:

| REV 3 STAGE | HERE | NOTE |
| --- | --- | --- |
| 0 Answer the specification queries | Partly | docs/uG24_Specification_Queries_ANSWERED.md records what the toolchain settled by construction and what still needs the vendor. The memory map (A1, A6), the DPTR convention (B4), the indirect-jump semantics (D5) and the shipped multiply/divide configuration (G5) remain genuinely open — and D5 is why O1 is still open. |
| 1 Get something that executes | Done | Rev 3 calls this the highest-value single action on the list, and it is right. ug24sim.c was written before the backend was trusted, and it is what turned every silent wrong-code bug into a visible failure. |
| 2 Rebuild the MC layer, encodings first | Done | All 51 encodings from the spreadsheet, each with a -show-encoding test and a round-trip test. |
| 3 Register pairing, then instruction selection | Done | Byte register file, eight pairs, DPTR/W/SFRs correctly aliased and reserved, post-RA pseudo expansion, shift expansion. |
| 4 Frame lowering, calls and the ABI | Mostly | Prologue, epilogue, real frame offsets, stack arguments and register masks all done. The Clang ABIInfo for aggregates is the piece still missing → O5. |
| 5 Relocations and linking | Mostly | Fixups, relocations and linking all correct and verified on a linked binary. Branch relaxation is the gap → O4. |
| 6 Runtime and startup | Mostly | Helpers, crt0 and the linker script are done. Vector table and the interrupt attribute are not → O8. |
| 7 Configuration matrix | Not started | Depends on G5 → O7. |

THE ONE THING TO CHANGE, AGREED Rev 3 ends by saying the 48-item checklist should not be reused as acceptance criteria, because its conditions are satisfiable by broken code — item 40 passed on a binary whose jumps the linker had just overwritten. That is correct and it is the most useful sentence in the document. The acceptance criterion used here is run-tests.sh: 40 programs compiled at five optimisation levels, executed on the simulator, and compared against expected results. A test that runs cannot pass on code that does not work.