Questions Regarding Hardware

- Is R14 or R15 the low byte of DPTR0?

  - R14 is the low byte; R15 is the high byte (DPTR0=R15,R14=X7).

- After MOV X7, SP with SP = 0xA55A, what values appear in R14 and R15?

  - R14 = 0x5A, R15 = 0xA5 (little-endian mapping).

- After PUSH RA, which byte is stored at each stack address?

  - SP-2 receives RA7:0 (low byte); SP-1 receives RA15:8 (high byte).

  - At the new SP (SPnew=SPold-2): SP=RA7:0 and SP+1=RA15:8.

- Is [SP] based on pre-decrement or post-decrement SP?

  - Hardware ISA: Post-decrement on PUSH (Rs→SP,SP←SP-1) and post-increment on POP (Rd←SP,SP←SP+1), implementing an Empty Descending stack.

  - Compiler ABI: Configured as Pre-decrement (Full Descending), where SP points to the last occupied byte.

- For a self-branch, does JR 0 branch to itself or to the following instruction?

  - The following instruction. Hardware executes PC←PC+1+i10; when i10=0, PC←PC+1 (acts as a NOP). A self-branch requires i10=-1 (0x3FF).

- Does JA 0x0004 address instruction memory using byte address 0x0004 or word address 0x0004?

  - Byte address 0x0004 in ELF files and assembler symbols.

- In instruction memory, which byte is fetched first for instruction word 0xA503?

  - 0x03 (least significant byte at address 2N; 0xA5 is at 2N+1).

- What byte order does PUSH RA use?

  - Little-endian (RA7:0 at lower address, RA15:8 at higher address).

- What byte order does instruction fetch use?

  - Little-endian (instruction bits 7:0 at address 2N, 15:8 at 2N+1).

Questions Regarding Software

- What are the sizes of char, short, int, long, and long long?

  - char: 1 byte (8 bits)

  - short: 2 bytes (16 bits)

  - int: 2 bytes (16 bits)

  - long: 4 bytes (32 bits)

  - long long: 8 bytes (64 bits)

- What is the pointer size?

  - 2 bytes (16 bits).

- What are the alignment requirements?

  - Data: 1-byte aligned (byte-aligned for all scalar types and structs).

  - Code: 2-byte aligned (all instructions and function entries).

- Which registers are used for function arguments?

  - R0, R1, R2, R3 (up to 4 bytes; excess arguments pass on the stack).

- Which registers are used for return values?

  - 8-bit: R0

  - 16-bit: X0 (R0 low, R1 high)

  - 32-bit: X0:X1 (R0:R3)

  - Structures >4 bytes: Caller allocates stack space and passes a pointer in R0:R1.

- Which registers are caller-saved and which are callee-saved?

  - Caller-saved (scratch): R0, R1, R2, R3, R4, R5.

  - Callee-saved (preserved): R6, R7, R8, R9, R10, R11.

  - Reserved: R12:R13 (DPTR1), R14:R15 (DPTR0), PC, RA, PSW, SP.

- What is the stack layout?

  - Full descending: Incoming arguments → Saved RA → Saved FP (optional) → Callee-saved registers (R6–R11) → Local variables/spills → Outgoing arguments (SP).

- What are the rules for passing variadic arguments?

  - Fixed arguments pass in R0–R3; variadic arguments (...) pass on the stack (or callee spills R0–R3 contiguously).

Project / Compiler Decisions

- What private ELF machine number should be used?

  - 0xBA51 (BASIL) or 250 (EM_UG2408 / 0x00FA).

- What relocation numbers should be used?

  - 0: R_UG2408_NONE

  - 1: R_UG2408_16 (16-bit absolute)

  - 2: R_UG2408_PCREL10 (10-bit branch relative)

  - 3: R_UG2408_8 (8-bit immediate)

  - 4: R_UG2408_LO8 (lower 8 bits of symbol)

  - 5: R_UG2408_HI8 (upper 8 bits of symbol)

- What are the meanings/semantics of each relocation?

  - R_UG2408_NONE: No-op.

  - R_UG2408_16: Write16leS+A.

  - R_UG2408_PCREL10: Offset=S+A-P+1; verify -512≤Offset≤511; insert into instruction bits 15:6.

  - R_UG2408_LO8: Write8S+A & 0xFF.

  - R_UG2408_HI8: Write8S+A≫8 & 0xFF.

- What linker script and memory layout should be used?

  - ROM/Flash: 0x0000 – 0x7FFF (32 KB for .vectors, .text, .rodata).

  - RAM/SRAM: 0x8000 – 0xFFFF (32 KB for .data, .bss; Stack top = 0xFFFF).

- What target triple should be used: is it ug24-unknown-elf?

  - Yes: ug24-unknown-elf